Live Boiler/Furnace
Transient Simulator

Short Teaching Presentation

Purpose: A compact guide to what the simulator represents, what students can learn from it, and where its educational limits are.

Mohsin Mohd Sies, HiREF, UTM - 2026

1. What It Is

A reduced-order, interactive teaching model of a fired boiler and its controls

A live transient simulator linking furnace combustion, heat transfer, steam generation, drum level, control action, alarms and selected faults.

A UTM-branded desktop PyQt application with a two-second startup splash, dashboard logo, adjustable inputs, virtual instrumentation, guided demo mode, case loading and CSV logging.

A systems-level learning environment: students observe how changes on the fuel/air side propagate into steam-side and control-system behaviour.

It represents a generic industrial drum boiler rather than one named manufacturer, burner, boiler geometry or power station.

Teaching role: It sits between hand calculations and high-fidelity plant simulation: more dynamic and integrated than a worksheet, but intentionally simpler than engineering design software.

2. What It Does

Observable behaviours and student interactions

Accepts gaseous, liquid, solid and user-defined fuels using elemental composition, molecular weight, heating value, moisture, nitrogen and ash inputs.

Calculates stoichiometric air, excess-air ratio, equivalence ratio, heat release, wall/stack losses, boiler heat capture and fuel-supported steam capacity, with a live Energy Sankey view of the heat balance.

Displays virtual flue-gas indications for O2, CO2, CO, NOx, unburned hydrocarbons, stack temperature, water dew point and dew-point margin.

Simulates steam temperature, feedwater and steam flow, corrected drum mass inventory, separate transient shrink/swell indication, spray attemperation and single-, two- or three-element drum-level control.

Provides coordinated firing, demand and fuel-quality steps, trips and alarms, plus a dedicated Fault injection box for transmitter faults, air leakage, poor mixing, fouling, wet fuel, NOx excursions and firing-demand mismatch.

3. What It Does Not Do

Important boundaries of interpretation

It does not perform computational fluid dynamics, burner aerodynamics, flame-shape prediction, furnace radiation-field solution or detailed tube-by-tube heat transfer.

It is not a complete or standards-conforming IAPWS-IF97 property package; the water/steam functions are simplified teaching approximations.

NOx and unburned-hydrocarbon outputs are trend models for learning, not regulatory emissions predictions or compliance calculations.

Fuel surrogates and user mixtures are idealised; detailed kinetics, pollutant chemistry, ash chemistry, slagging, corrosion and deposit growth are not resolved.

It is not suitable for boiler design, equipment sizing, safety analysis, licensing, performance guarantees or operator certification.

Interpretation rule: Use the simulator to explain directions, interactions and control consequences—not to claim plant-specific numerical accuracy.

4. Physics Included

The main physical relationships represented

Combustion stoichiometry from fuel C-H-O-S composition, including theoretical oxygen demand, actual air, lambda and phi.

Incomplete combustion through a carbon-to-CO fraction influenced by air deficiency, combustion completeness and burner mixing quality.

Boiler energy balance using lower heating value, combustion efficiency, wall loss, stack loss and effective heat capture; the same terms drive the live Energy Sankey diagram.

Steam-side enthalpy balance across economiser, evaporator and superheater duties, including spray-water mixing for outlet steam temperature.

Dynamic lags for major measured and process variables, PI control loops, anti-windup, actuator response and density compensation for differential-pressure flow measurement.

Corrected drum mass inventory plus a separate transient shrink/swell indication driven by changes in steam release, with a delayed low-fuel-for-feedwater warning when inventory is expected to rise.

Post-furnace air dilution, analyzer lag/noise, water-vapour dew point, fouling effects, alarms and master fuel trip logic.

5. Physics Simplified

Where reduced-order approximations replace detailed models

The entire furnace and boiler are treated as lumped components; spatial distributions of temperature, heat flux, species and residence time are not solved.

Water/steam properties use compact correlations for saturation temperature, enthalpy and specific volume rather than full IF97 region equations.

Heat duty is divided into fixed economiser/evaporator/superheater fractions instead of being calculated from geometry and local heat-transfer coefficients.

Flame temperature is a bounded proxy, while thermal, fuel and prompt NOx are transparent empirical trend expressions.

CO and UHC formation use heuristic breakthrough and mixing relationships, not chemical-kinetic mechanisms.

Time constants and controller gains are selected for visible teaching transients, not identified from a particular operating boiler.

6. Pedagogical Decisions Taken

Design choices made to improve teaching clarity

Fast, visible transients: the model uses a 0.25 s simulation step and deliberately short lags so students can see cause and effect within a class session.

Trend-first design: rolling plots use simulated time before present on the x-axis; alarm tiles, emissions plots, a flue-gas map and the Energy Sankey make relationships visible.

O2 is not treated as sufficient by itself: poor-mixing and post-furnace-leak cases show why CO, CO2 and operating context must be interpreted together.

Controls are layered progressively: single-, two- and three-element drum-level strategies can be selected or automatically degraded when transmitters become unreliable.

Fault cases are grouped in a dedicated dashboard box and map only to represented behaviours, helping prevent conclusions from unsupported physics.

A guided demo restores predictable initial conditions and sequences stable operation, demand response and representative combustion/heat-transfer faults; coordinated firing normally follows steam demand unless a training case disables it.

Simplified equations are intentionally transparent so instructors can discuss assumptions, limitations and competing operational objectives.

Teaching Takeaway

How the simulator should be framed in class

Best use: Ask students to predict the direction of change, run a disturbance, compare the trends, diagnose the cause, and justify an operator response.

The strongest learning outcomes are systems thinking, transient interpretation, combustion diagnosis, control-mode comparison and recognition of misleading measurements.

Numerical outputs should be discussed as internally consistent teaching estimates, not as validated performance data for a real boiler.

Suggested discussion prompt: “Which measurement changed first, which controller responded, and which apparent symptom could lead to a wrong diagnosis?”

Basis: Live Boiler/Furnace Transient Simulator v1.2.0 source code, reviewed August 2026.